END OF JOB REPORT

MIT LOGGING

TEMBUNGO-BRAVO

B02S

| Prepared by | Reviewed by | Received by |
| --- | --- | --- |
| Clement Emang Field Engineer | Amrie Zulkifli Account Manager | Aina Well Integrity Engineer |

Contents

1.	CLIENT DETAILS	2

2.	JOB DETAILS	2

3.	PERSONNEL DETAILS	2

4.	INTRODUCTION	3

5.	OBJECTIVES	4

6.	WELL SCHEMATIC	4

7.	SEQUENCE OF EVENT/OPERATION SUMMARY	6

8.	SUMMARY	6

9.	ATTACHMENT	7

# CLIENT DETAILS

CLIENT					: PCSB SKA

FIELD					: TEMBUNGO

PLATFORM/RIG			: TBG-BRAVO

WELLS NAME				: B02S

ENGINEER IN CHARGE		:

COMPANY MAN			: Julius, Zaid

# JOB DETAILS

MAIN OBJECTIVE			: MIT LOGGING

MOBILIZATION DATE			: 22/9/2023

TYPE OF SERVICES			: EMITE (MEMORY MIT LOGGING)

CONVEYANCE				: Deleum Slickline Unit

# PERSONNEL DETAILS

DIVISION				: EMITE

FIELD ENGINEER			: Clement Emang

3RD PARTY COMPANY			: DELEUM

3RD PARTY PERSONNEL		: Douglas, Zaidie, Mazlani

FIELD OPERATION SUPPORT		: Faris Firdaus

ACCOUNT MANAGER			: Amri Zulkifli

# INTRODUCTION

- The proposed work scope for TBG-BRAVO is to perform MIT logging using MIT24F to inspect the tubing internal condition and integrity by analysing any corrosion, erosion or physical damage prior to Plug and Abandonment.

| Mechanical | MIT24F |
| --- | --- |
| Outside Diameter | 1-11/16” |
| Make up Length | 50.6” |
| Housing Material | Inconel 718 / Stainless Steel 17-4 |
| Finger Lengths | 7” |
| Number of Fingers | 24F |
| Radial Accuracy | ±0.02” |
| Radial Resolution | 0.003” |
| Measurement Range | 1-3/4”(Min) 7”(Max) |
| Circumferential Spacing | 0.23”(Min) 0.92”(Max) |
| Finger Tip Radius | 0.05” |
| Finger Tip Width | 0.063” |
| Finger Contact Force | 0.75lbs(Min) 1.25lbs(Max) |
| Logging Speed | 30ft/min(Min) 60ft/min(Max) |
| Environmental |  |
| Pressure Ranges | Up to 15,000 psi |
| Temperature Rating | 177°C (351°F) |
| Electronic |  |
| Current Consumption | Logging <30mA Motor <400mA |

# OBJECTIVES

▪ To provide MIT survey to inspect the tubing internal condition and integrity by analysing any corrosion, erosion or physical damage prior to Plug and Abandonment.

PLANNED

| Run | WELL | RUN | INTERVAL |
| --- | --- | --- | --- |
| 1 | B02S | MIT LOGGING | 3576 ft-MDTHF to surface |

# WELL SCHEMATIC

WELL NAME				: B02S

WELL TYPE				: Dual Producer

TUBING DETAILS			: 2-7/8”

CASING DETAILS			: 9-5/8”

CITHP (PSIA)				: -

A-ANNULUS PRESSURE		: N/A

MAXIMUM DEVIATION 		: 36.0 DEG

FIELD ENGINEER			: CLEMENT EMANG

CREW	: -

3RD PARTY REP / CONVEYANCE	: DOUGLAS, ZAIDIE, MAZLANI

# SEQUENCE OF EVENT/OPERATION SUMMARY

ACTUAL

| Run | WELL | RUN | INTERVAL | STATUS |
| --- | --- | --- | --- | --- |
| 1 | B02S | MIT LOGGING | 3380 ft-MDTHF to 20 ft-MDTHF | COMPLETED |

| Date | Description/Operation Summary |
| --- | --- |
| 19/10/2023 | - Deleum run TCC but encounter HUD at 3390 ft-MDTHF. Inform WSS. After discussion with WSS/town, decided to run MIT until 10 ft above HUD. New interval for MIT logging is 3380 ft-MDTHF to 20 ft-MDTHF - Perform MIT run from 3380 ft-MDTHF to 20 ft-MDTHF - Toolstring: RS + Swivel + 5ft stem + 3ft stem + KJ + ABM + UMT + CCL + PKJ + PKJ + PRC + MIT24 + PRC + BUL - RIH to target depth with 120 ft/min, slow at accessories and pick up weight every 1000ft. - Reach target depth. Wait for finger to open. - Log up 30 ft/min - Reach target depth. Wait for finger to close. - POOH to surface - Disconnect tool from slickline toolstring - Disconnect battery - Download data. |

# SUMMARY

- Successfully achieve all the objectives.

- Depth control and QC were performed based on TRSCSSV as reference. Minimum ID at the seal bore reading was in between tolerance

- All fingers working good and fully open throughout the logging and overall log quality is good and interpretable

- Field Quick Look Report shows that the MIT24F detects light to moderate corrosion in the range between 10%-30%

- There is no possible leak that exceeds 70% of penetration in tubing wall which classified as possible hole in the well.

# ATTACHMENT

| NO | DOCUMENT |
| --- | --- |
| 1 | Daily Offshore Report |
| 2 | Field Quicklook |
| 3 | MIT Log |