| Possible Contingencies in CTU Operations |  |
| --- | --- |
| 1 | Stripper Failure (Coil Tubing - Single Stripper) |
| 2 | Stripper Failure (Coil Tubing - Dual Stripper) |
| 3 | Pinhole (in Coil Tubing) at surface |
| 4 | Coil Tubing parted at surface |
| 5 | Coil Tubing parted down-hole |
| 6 | Coil Tubing Collapse |
| 7 | Coil Tubing Runaway while RIH |
| 8 | Coil Tubing Runaway while POOH |

Increase Hydraulic Pressure on Stripper

Leak Stopped

NO

YES

Assess, Discuss further Operation

Stop Pipe Movement & Circulation

Apply Injector Brake

Shut In Well at Choke & Wing Valve if Open

Close Slip & Pipe Rams

Bleed off above Pipe Rams

Leak Stopped

YES

Replace Stripper Rubber & Pressure Test

Assess, POOH &

Secure Well

Shallow

NO

CT Depth

Deep

Pick up enough to ensure pipe drops clear of X-Mas Tree when cut

Equalize Pressure.

Open Pipe Rams & Slip Rams

Continue Further Operations

Operate Shear/Seal Rams of Shear Seal BOP (If Fitted) or Shear Rams of Quad BOP. If Shear Rams of Quad BOP are operated, pick up the pipe enough to ensure the Blind Rams can be closed to seal the well bore

Secure Well & Repair Stripper

Assess, Discuss to Fish Coil

YES

Leak Stopped

NO

Close Lower

Stripper

Assess,

Discuss further Operation

YES

Shallow

Leak Stopped

CT

POOH &

Repair Upper Stripper

NO

Deep

Circulation

Continue Further Operations

Bleed off above Pipe Rams

Equalize Pressure.

Open Pipe Rams & Slip Rams

YES

Leak Stopped

Replace Stripper

Rubber & Pressure Test

NO

Increase Hydraulic Pressure on Upper Stripper

Stop Pipe Movement &

Apply Injector Brake

Shut In Well at Choke & Wing Valve if Open

Close Slip & Pipe Rams

Pick up enough to ensure Pipe drops clear of X-Mas Tree when cut

Operate Shear/Seal Rams of Shear Seal BOP (If Fitted) or Shear Rams of Quad BOP. If Shear Rams of Quad BOP are operated, pick up the pipe enough to ensure the Blind Rams can be closed to seal the well bore

Secure Well & Repair Stripper

Assess, Discuss to Fish Coil

Pinhole (in Coil tubing) at surface

Pinhole above While RIH Pinhole above While POOH Hole formed in the CT downhole

Monitor CT pressure & observed Check Valves are holding. Do not allow pressure to bleed below collapse pressure for CT

Check Valves Holding

YES

POOH until Leak point is back on Reel

NO	NO

Severe

Leak

YES

Close Slip & Pipe Rams

POOH

Displace Reel to Water if there is hazardous fluid in Reel (e.g.

Acid)

Operate Shear Rams to Cut Pipe

Close Blind Rams

POOH & Replace Reel

Rig down Injector & remove Leaking Pipe

Circulate well to Kill Fluid through pipe left in-between the blind & pipe rams

Rig up Injector (with New Pipe) to fish Old Pipe

Attempt to spool excess coil back on to the reel to prevent it from swinging around. At the same time use the Injector to pull the other end back onto the Gooseneck

Stop injector and close the pipe and slip rams. Manually lock the rams if safe to do so

Check Valves Not Holding Or Danger of Fluid Release

NO

Monitor SIWHP & Prepare for Fishing Job

YES

Operate Shear Rams to Cut Pipe

Monitor whilst crew member stands by ready to follow the designated cutting method if leak re-occurs

Close Blind Rams

Rig down Injector & remove Leaking Pipe

Circulate well to Kill Fluid through pipe left in-between the blind & pipe rams

Rig up Injector (with New Pipe) to fish Old Pipe

Parted tubing will be indicated by a

loss of weight and reduction in circulating pressure

Maintain low rate circulation at all

times to prevent hydrocarbons from entering the coil

Estimate length of coil left in the well

from pick-up and hanging weight

Pull out of hole slowly until it is

estimated that the end of the reel is just above the Swab Valve

Attempt to close the swab by counting turns. If not close (coil

across the valve) open the valve again & pick up CT equal to or a little shorter than the length of the riser and try the valve again.

Repeat until it is possible to close the valve.

Close Master Valve

Bleed off Riser Pressure &

Rig Down

Prepare for

Fishing Operations

Indications of CT Collapse are:-

- Increase in pump pressure

- Increase in tension measured on the stripper bushing

- Gripper is slipping

- Increase in tubing weight

Lower the coiled tubing into the hole

and place good pipe across the BOP rams

Close and lock the slip & pipe rams

Test slip rams by slowly

reducing the load on the injector

Kill the Well

Assess & Discuss to Recover Coil

| Well Services Emergency Response Plan | Page 21 of 24 | Revision 1.0 |
| --- | --- | --- |

Increase the traction system pressure

YES

CT Stopped

NO

Apply the "Emergency Traction" pressure from the priority hydraulic system. Keep the injector control in the “Run in Hole” position

Assess &

Discuss further Operation

YES

CT Stopped

NO

Increase the hydraulic pressure to the maximum setting in the stripper system

Assess &

Discuss further Operation

YES

CT Stopped

NO

Assess &

Discuss further Operation

area

Move Non-essential personnel to safe Area

Operator to Evacuate to safe

When CT has hit the bottom of the well, close the pipe & slip

Determine damage in

CT downhole by pumping

YES

Assess &

Discuss Options

NO

Repair Injector

rams

System

Open Pipe & Slip rams

Slowly POOH

### CT Runaway while Pulling out of Hole

Move Non-essential personnel to safe Area

Increase the traction system pressure

YES

CT Stopped	Assess &

Discuss further

Operation

NO

Apply the "Emergency Traction" pressure from the priority hydraulic system. Keep the injector control in the “Pull Out of Hole” position

YES

CT Stopped	Assess &

Discuss further

Operation

NO

Increase the hydraulic pressure to the maximum setting in the stripper system

YES

CT Stopped	Assess &

Discuss further

Operation

NO

Operator to Evacuate to safe area

When CT has been thrown out of the hole, assess & if feasible, close the blind rams and wellhead valves